ALSTEF GROUP

Support Services Team

# Queenstown Airport – BHS Performance Review

Period covered: July 2026 (1–31 July, 31 days)

Prepared by: Tiaan Botha, Alstef Group

# Executive Summary

- The system processed 119,227 bags through the main outbound line in July (as asked excluding oversize), plus a further 10,445 through the oversize line, averaging just under 3,850 bags per day.

- Of those, 3,023 bags (2.5%) were ultimately rejected after full screening, this is the number that matters operationally, and it held steady all month.

- A much larger share (34.1%) gets flagged at the first automated scan, but the vast majority of those clear on the second, more thorough scan. This is normal, expected behaviour for a two-tier EDS system, not a fault.

- Bag tracking reliability was strong: 1,756 lost-bag events against 691,736 tracked scans, a 0.25% rate. One notable spike on 14 July is worth a mention (see Observations).

- Nothing in the data points to a system health or security issue, the trends are consistent with normal within-month volume swings.

# 1. Total Throughput (excluding oversize)

You asked for total bag throughput excluding oversize bags. The GlideView main-line counter (T101) is the right number for this, it tracks every bag on the primary outbound system, separate from the oversize induct (OS104).

| Metric | Value | Note |
| --- | --- | --- |
| Main line throughput (T101), July 2026 | 119,227 bags | Excludes oversize |
| Oversize line (OS104), July 2026 | 10,445 bags | Separate system |
| Average daily throughput (main line) | ~3,846 bags/day | Range: 2,660 – 4,933 |

# 2. Reject Bags

Queenstown's EDS screening runs in two stages.

- Level 1 (automated CT scan): flags any bag it can't clear with full confidence and sends it for a second look. In July, 40,174 bags (34.1% of all screened bags) were flagged this way.

- Level 2 (secondary screening): re-examines everything L1 flagged. Of those, 36,445 cleared and only 3,023 (7.5% of L1-flagged bags, or 2.5% of total throughput) were confirmed as genuine rejects.

For reporting purposes, the 3,023 figure (2.5% of throughput) is the meaningful "reject bag" number, it reflects bags that actually required manual handling after full screening, not bags that simply needed a second automated look.

| Metric | Value | Note |
| --- | --- | --- |
| L1 flagged for re-screen | 40,174 bags (34.1%) | Sent to L2, not a final reject |
| L2 cleared after re-screen | 36,445 bags | 90.8% of L1-flagged bags |
| L2 confirmed reject (final) | 3,023 bags (2.5% of throughput) | The operational reject figure |
| L2 timeout | 706 bags | Rescreen not completed in time |

# 3. Comparative Percentage

Reject rate stayed remarkably consistent across the month, daily final-reject rate ranged 1.5%–3.3%, averaging 2.5%, with no sustained upward or downward trend. The L1 auto-flag rate did drift up slightly in the second half of July (33.6% average in the first 15 days vs. 34.7% in the last 16), but the L2 clear rate absorbed it, so the end result for the terminal was stable.

# Trend Charts

Daily throughput against L1 and L2 (final) reject volumes, July 2026:

Reject rate as a percentage, July 2026, L1 auto-flag rate vs. final (L2) reject rate:

Bag tracking reliability, lost-bag events per day, July 2026:

# Snapshot: Final Week of July (25–31 July 2026)

A closer look at the most recent full week within the reporting month, useful if Sean wants to compare against next month's opening week.

| Metric | Value |
| --- | --- |
| Total throughput (main line) | 25,493 bags |
| L1 flagged for re-screen | 8,759 bags (35.4%) |
| L2 confirmed reject (final) | 678 bags (2.66% of throughput) |
| Lost-bag events | 219 (0.15% of tracked scans) |

# Observations & Discussion Points

- 14 July lost-bag spike: 187 lost-bag events that day (vs. a ~52/day average for the rest of the month), concentrated around T110PC.

- Slight L1 auto-flag drift: the automated reject rate crept from a 33.6% average in the first half of July to 34.7% in the second half. Since the final (L2) reject rate stayed flat, this looks like normal scan sensitivity variation rather than a developing fault, but it's worth keeping an eye on if it continues climbing into August.

- Oversize volumes trending up: oversize bag counts roughly doubled across the month, from a ~209/day average in the first week to ~416/day in the last week, likely seasonal, but useful context if Sean is looking at capacity.