17-19 Patiki Rd, Avondale, Auckland, 1026, New Zealand

# Weekly Support Tickets Report:

24th August to 30th August 2026

# Total Amount of Calls: 23

# Total Amount of Direct Contacts: 2

| Africa: 0 | Australia: 1 | Bermuda: 2 |
| --- | --- | --- |
| Canada: 10 | Jamaica: 0 | New Zealand: 8 |
| United States: 4 | Parcels: 0 |  |

# Excustive aSumary:

•  23 Calls received this week

•  2 Direct Contacts

•  69.6% of calls received were after hours

- Most frequent sites for Airport and Parcel

- YLW_APAC accounted for 24% of calls

# Africa: 0 tickets

# Australia: 1 ticket

Adelaide Airport ADL

15200	(29/08/2026 09:34)

Reported: The newly added SL111c PE2 began causing an operational or system error immediately after installation.

Remedy:  A request was made to disable SL111c PE2 directly within the PLC/software code to resolve the disturbance.

# Bermuda: 2 Tickets

Bermuda Airport BDA

15194	(28/08/2026 04:36) & 15195 (28/08/2026 11:51)

Reported: Marcus at Bermuda Airport reported a BHS system issue following a failed ASI gateway on MU1.

Findings: CB1 network switch identified as causing intermittent simultaneous dropouts across RE, CB1, and MU1 panels. Switch replacement recommended.

Remedy: The diagnostic port IP address wasn’t set, and possibly the other port gateway was a typo. The weird thing was, even shutting down the panel or the gateway still influenced the other ones, dropping them out.

# Canada: 10 Tickets.

Kelowna Airport YLW

15179	(24/08/2026 03:3) & 15180 (24/08/2026 04:09:24) &

15181	(24/08/2026 04:26)

Reported: The whole TC2 line before the Pathfinder; all sensors were off and Estops were pressed.

Findings: Checked the PLC; all the I/O were looking good.

Remedy: Initial findings found a fuse fault at TC2 panel. After changing the fuse, it blew again. An electrician was sent out to site, and eventually it was determined the issue was with one of the scales. After replacing the fuse, the system was operational.

15189	(27/08/2026 07:46) & 15190 (27/08/2026 09:20)

Reported: TC2 was completely down with multiple SCADA faults, including fuse and E-stop faults.

Findings: No physical fuses or PLC I/O faults were found. The issue was traced to a loss of 24VDC supply, which caused the E-stop faults as a secondary effect.

Remedy: The faulty 24VDC power supply was replaced, clearing all faults and restoring TC2 to normal operation.

Direct Email (28/08/2026 08:14)

Reported: Inbound baggage carousels 1 & 2 at YLW are displaying as "offline" due to a communication fault.

Findings: IPs 192.168.7.51 through .53 were successfully pinged from both the Glidepath server and a local laptop on the same subnet, ruling out a basic physical or network-layer drop.

Remedy: Review the IGS driver configuration and software service status, given the previous occurrence of this same fault.

Edmonton YEG

15192	(27/08/2026 23:06) & 15193 (28/08/2026 03:27)

Reported: BagSort again where the bags not able to sort.

Remedy: Contacted France to investigate the issue. Timothée BOURDEAU is looking into the issue.

Direct Email (26/08/2026 06:55)

Reported: On 20 August at 5:58 PM, ITD reported that a passenger had 2 checked bags, while the ITD booth screen displayed 4 bags. All 4 bags were recalled.

Findings: The airline system confirmed only 2 checked bags, and only 2 bags arrived in Secondary. The recall records showed 4 baggage tags recalled between 6:25:06 PM and 6:25:33 PM, with no last scan location recorded.

Remedy: A bug in the system allows bags with received BSM delete messages to still appear at the primary inspection station, but only when a passenger's boarding pass is scanned. A fix is being worked on by Scott Y and due to be deployed.

15197	(28/08/2026 16:31)

Reported: BagSort Communication failures.

Remedy: Escalated to Timothee from AA.

# Jamaica: 0 tickets

# New Zealand: 8 Tickets

Queenstown Airport ZQN

15184	(25/08/2026 16:53) & 15185 (25/08/2026 17:07)

Reported: Roller door is closed but the Prox activated.

Remedy: Sent out Shane, to site 25/08/2026.

15201	(29/08/2026 14:01)

Issue: T110 conveyor repeatedly tripped (8–10 times) on a pulse wheel fault, forcing a 5-minute system shutdown each time.

Findings: A logic timer conflict in the code. The motor failed to run (a motor feedback issue), which naturally produced no pulse signals. Because the pulse wheel fault timer (1.0s) was set shorter than the motor feedback timer (4.5s), the PLC falsely reported a pulse wheel failure instead of the true motor fault.

Remedy: Reduced the motor feedback fault timer to 0.7s so future motor failures are correctly identified. The belt is currently running normally.

Auckland Domestic Airport AKL

15196	(28/08/2026 13:03)

Reported: Site reported a plough offline at T111

Findings: First started at 12.45pm and since then the online light hasn't come back.

Remedy: Resolved on site by baggage handling technicians.

Wellington Intl. Airport WLG

15183	(25/08/2026 05:56)

Reported: Transfer line had a critical fault where the voltage monitor relay is tripped.

Findings: It's just a monitoring relay and shouldn't fault the system therefore, I masked it so that it won't cause critical fault.

Remedy: Site electrician reset or replaced the relay.

15191	(27/08/2026 14:42)

Reported: Wellington Airport reported a “Voltage Monitor Relay Fault” alarm on TC1 SCADA. The issue had been occurring since 25 August but was not impacting operations.

Findings: Remote checks of SCADA showed no active TC1 alarms at the time of investigation. The site advised that an electrician attended shortly after the call and made an adjustment to the voltage.

Remedy: The voltage adjustment resolved the alarm, and the system returned to normal operation.

Dunedin Airport DUD

15198	(28/08/2026 18:33) & 15199 (29/08/2026 05:04)

Reported: Motor Fault on Outbound Carousel.

Remedy: Grant from Propower was sent out on the 29th.

USA: 4 TICKETS

Boise Airport BOI

15178	(24/08/2026 01:35)

Reported: SL2 ATR is currently showing "Both Units Active Fault".

Findings:  The Active signal hardwired to the remote I/O at HBS7 panel, Rack 1, module 2, bit 30 and bit 31.

Remedy: There should only one be active controller. Both should not be active at the same time. Once a full power cycle of this ATR was completed. This reset the redundancy and only one controller was active, and the fault was be cleared.

San Francisco Intl. Airport SFO

15186	(26/08/2026 01:32) & 15188 (27/08/2026 02:52)

Reported: Bag drop 39 had a dimension camera disconnected fault.

Remedy: Restarted the bag drop fixed the issue.

15187	(26/08/2026 06:44)

Reported: All ATR cameras and Dimensioning cameras were disconnected

Remedy: just rebooted the SBD with "sudo reboot now"

# Parcels: 0 tickets

Warm regards,

ALSTEF Group
Support Services Team